TARBOC

New private model 295W beam lamp

Instructions for use

Version number: MF 3.0

Precautions and Installation

Maintenance

- This lamp should be kept dry and avoid working in a humid environment.

- Intermittent use will effectively extend the life of the lamp.

- In order to obtain good ventilation and lighting effects, pay attention to cleaning the fan, fan net and lens frequently.

- Do not use alcohol and other organic solvents to wipe the lamp shell, so as not to cause damage.

Statement

This product in the factory into, the performance intact, complete packaging. All users should strictly follow the warnings and operating instructions stated above,

Any damage caused by misuse is not within the company's guarantee, and the failure and problem caused by ignoring the operation manual are not within the scope of the dealer's responsibility.

This manual is subject to technical changes without notice.

Product considerations

- In order to ensure the service life of the product, this product should not be placed in a damp or leaking place, and should not work in an environment where the temperature exceeds 60 degrees.

- Do not place this product in a place that is easy to loosen or vibrate.

- In order to avoid the risk of electric shock, the maintenance of this product please seek professional maintenance.

- When the bulb is used, the power supply voltage change should not exceed ± 10%. If the voltage is too high, the life of the bulb will be shortened. If the voltage is too low, the light color of the bulb will be affected.

- After the power is off, it takes 20 minutes to use the lamp to fully cool down before it can be powered on again.

- In order to ensure the normal use of this product, please read this instruction carefully. Signal line connection (DMX)

Use RS-485 cable that meets specifications: shielded, 120ohm characteristic impedance, 22-24 AWG, low capacitive reactance. Do not use microphone cables or cables with different specified characteristics. The terminals must be connected using a 3-or 5-pin XLR type male/female connector. (1/4W minimum).

Important note: The wires cannot contact each other or the metal housing.

Figure 1DMX signal line connection diagram

Installation of lamps

Luminaires can be placed horizontally, slanted and hung upside down. Be sure to pay attention to the installation method when hanging obliquely and upside down.

As Fig. 2Inverted lamp of shown, in the lamp positioning, to ensure that the installation site of stability, in the reverse hanging installation,

It must be ensured that the lamp does not fall on the support frame, and the safety rope needs to pass through the support frame and the lamp handle for auxiliary hanging,

To ensure safety and prevent lamps from falling and sliding.

During the installation and commissioning of the lamps, pedestrians are prohibited from passing below, and the safety ropes are regularly checked for wear and the hook screws are loose.

If the hanging installation is not stable, resulting in all the consequences of falling lamps and lanterns, our company does not bear any responsibility.

Fig. 2Inverted lamp of

| Main Menu | Submenu | Menu function description |  |
| --- | --- | --- | --- |
| 1. System Settings | 1. DMX Address | Modify DMX address code up or down (address code can be selected from 001-512), confirm key to save, default 001. |  |
|  | 2. Encoder | Up or down to modify the encoder to open/close, confirm the key to save, the default to open. |  |
|  | 3, X reversal | Up or down to modify the X reverse open/close, confirm the key to save, the default closed. |  |
|  | 4. Y reversal | Up or down to modify the Y reverse open/close, confirm the key to save, the default closed. |  |
|  | 5, no signal | Hold/Clear |  |
|  | 6, boot bright gun bubble | ON/OFF |  |
|  | 7. Reset | ON/OFF |  |
|  | 8. Advanced Settings |  |  |
| 2. Operation mode | 1. Operation mode | Up or down to select the operation mode DMX/voice control/automatic, confirm the key to save, the default DMX. |  |
|  | 2. Channel selection | Up or down select channel CH16.CH18 to confirm the key to save. Default CH16 |  |
| 3. Manual control | 1. Color wheel | Modify up and down values 000-255 default 000 |  |
|  | 2. Strobe | Modify up and down values 000-255 default 000 |  |
|  | 3. Dimming | Modify up and down values 000-255 default 000 |  |
|  | 4. Pattern plate | Modify up and down values 000-255 default 000 |  |
|  | 5. Prism 1 | Modify up and down values 000-255 default 000 |  |
|  | 6. Prism 1 Rotation | Modify up and down values 000-255 default 000 |  |
|  | 7. Prism 2 | Modify up and down values 000-255 default 000 |  |
|  | 8. Prism 2 Rotation | Modify up and down values 000-255 default 000 |  |
|  | 9, focusing | Modify up and down values 000-255 default 000 |  |
|  | 10. X-axis | Modify up and down values 000-255 default 000 |  |
|  | 11. X-axis fine adjustment | Modify up and down values 000-255 default 000 |  |
|  | 12. Y axis | Modify up and down values 000-255 default 000 |  |
|  | 13. Y-axis fine adjustment | Modify up and down values 000-255 default 000 |  |
|  | 14. XY speed | Modify up and down values 000-255 default 000 |  |
|  | 15. Atomization | Modify up and down values 000-255 default 000 |  |
|  | 16, colorful. | Modify up and down values 000-255 default 000 |  |
|  | 17. Bulb Control | Modify up and down values 000-255 default 000 |  |
|  | 18. Reset | Modify up and down values 000-255 default 000 |  |
| 4. Display Settings | 1. Language selection | Up or down to select Chinese/EN (English), confirm the key to save, the default Chinese. |  |
|  | 2. Inverted display setting | Select up or down to turn off/turn on the reverse display setting, confirm the key to save, and turn off by default. |  |
|  | 3, screen protection | Mode 1-Mode 4 |  |
|  | 4. DMX index | Mode 1-Mode 4 |  |
|  | 5. Backlight display | 010 |  |
| 5. Factory setting 4, | Please enter the password! Change up or down to 138, and then press the Enter key to enter the factory setup. 4, | 1. X-axis calibration | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 2. Y-axis calibration | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 3. Color wheel calibration | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 4. Solid diagram calibration | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 5. Strobe calibration | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 6. Strobe stroke | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 7. Strobe half step | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 8. Focus Calibration | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 9. Focusing stroke | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 10. Prism 1 stroke | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 11. Prism 1 Half Step | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 12. Prism 2 stroke | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 13. Prism 2 Half Step | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 14. Atomization stroke | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 15, atomization half step | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 16, colorful itinerary | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 17, colorful half step | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 18. Dynamic drawing calibration | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 19. Dynamic chart rotation calibration | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 20. Amplification Calibration | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 21. Enlarged stroke | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 22. Uniform light stroke | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 23, uniform light half step | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 24. Aperture stroke | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 25, aperture half step | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 26. CMY1 Calibration | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 27. CMY2 calibration | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 28. CMY3 calibration | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 28. Fine adjustment 1 | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 29. Fine adjustment 2 | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 30. Fine adjustment 3 | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 31. Fine adjustment 4 | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 32. X speed | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 33. X stroke | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 34. X current | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 35. Y speed | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 36. Y stroke | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 37. Y current | Modify parameters up or down (range 000-255), confirm key save, default 128. |
|  |  | 38. Fan speed | 7000 |
|  |  | 39. Time Clearing | Yes/No |
|  |  | 40, half power | ON/OFF |
|  |  | 41. Fan Control | ON/OFF |
|  |  | 42, Logo | 000 |

CH16 (Detailed Channel Table)

| Channel | Function | Channel Value |  |
| --- | --- | --- | --- |
| 1 | Color disc | 0 - 9 10 - 14 15 - 19 20 - 24 24 - 29 30 - 34 35 - 39 40 - 44 45 - 49 50 - 54 55 - 59 60 - 64 65 - 69 70 - 74 75 - 79 80 - 84 85 - 89 90 - 94 95 -99 100-104 105-109 110-114 115-119 120-124 125-129 130-134 135-139 140-149 150-203 204-255 | Color 1 Color 1 + Color 2 Color 2 Color 2 + Color 3 Color 3 Color 3 + Color 4 Color 4 Color 4 + Color 5 Color 5 Color 5 + Color 6 Color 6 Color 6 + Color 7 Color 7 Color 7 + Color 8 Color 8 Color 8 + Color 9 Color 9 Color 9 + Color 10 Color 10 Color 10 + Color 11 Color 11 Color 11 + Color 12 Color 12 Color 12 + Color 13 Color 13 Color 13 + Color 14 Color 14 Color 14 + Color 1 Forward flow (from fast to slow) Reverse flow (from slow to fast) |
| 2 | Strobe | 0 - 3 4 -103 104-107 108-207 209-212 213-251 252-255 | closed light Pulse strobe (from slow to fast) open light Effect strobe (slow to fast) open light Random strobe (slow to fast) open light |
| 3 | Dimming | 0-255 | From dark to light |
| 4 | pattern plate | 0 - 5 6 - 11 12 - 17 18 - 23 24 - 29 30 - 35 36 - 41 42 - 47 48 - 53 54 - 59 60 - 65 66 - 71 72 - 77 78 - 83 84 - 89 90-134 135-139 140-185 186-190 191-195 196-200 201-205 206-210 211-215 216-220 221-225 226-230 231-235 236-240 241-245 246-250 251-255 | Solid Figure 1 Solid Figure 2 Solid Figure 3 Solid Figure 4 Solid Figure 5 Solid Figure 6 Solid Figure 7 Solid Figure 8 Solid Figure 9 Solid Figure 10 Solid Figure 11 Fixed Figure 12 Fixed Figure 13 Fixed Figure 14 Fixed Figure 15 Forward flow (from fast to slow) Solid Figure 1 Reverse flow (from slow to fast) Figure 2 Jitter (from slow to fast) Figure 3 Jitter (from slow to fast) Figure 4 Jitter (from slow to fast) Figure 5 Jitter (from slow to fast) Figure 6 Jitter (from slow to fast) Figure 7 Jitter (from slow to fast) Figure 8 Jitter (from slow to fast) Figure 9 Jitter (from slow to fast) Figure 10 Jitter (from slow to fast) Figure 11 Jitter (from slow to fast) Figure 12 Jitter (from slow to fast) Figure 13 Jitter (from slow to fast) Figure 14 Jitter (from slow to fast) Figure 15 Jitter (from slow to fast) |
| 5 | Prism 1 | 000-127 128-255 | Prism 1 pop-up Prism 1 cut-in |
| 6 | Prism 12 rotation | 000-064 065-127 128-191 192-255 | Index Forward running water (from slow to fast Reverse flow (from slow to fast) Positive and negative turn (from slow to fast) |
| 7 | Prism 2 | 000-127 128-255 | Prism 2 pop-up Prism 2 Cut |
| 8 | atomization | 000-255 | 128-255 Atomizer Cut-in |
| 9 | Focus | 000-255 | Pattern clarity from far to near |
| 10 | X | 000-255 | Horizontal 540 degree scanning |
| 11 | X Trim | 000-255 | Horizontal 1.2 Trim |
| 12 | Y | 000-255 | Vertical 270 degree scan |
| 13 | Y Trim | 000-255 | Vertical 1.2 Trim |
| 14 | XY speed | 0-255 | From fast to slow |
| 15 | Colorful | 0-255 | 0-127 colorful mirror pops up 128-255 Colorful Mirror Cut In |
| 16 | On/Reset | 0-255 | 0-99 No effect 100-105 Turn off the bulb 106-199 No effect 200-205 light bulb 206-255 RESET |

Channel Table

CH18 (Detailed Channel Table)

| Channel | Function | Channel Value |  |
| --- | --- | --- | --- |
| 1 | Color disc | 0 - 9 10 - 14 15 - 19 20 - 24 24 - 29 30 - 34 35 - 39 40 - 44 45 - 49 50 - 54 55 - 59 60 - 64 65 - 69 70 - 74 75 - 79 80 - 84 85 - 89 90 - 94 95 -99 100-104 105-109 110-114 115-119 120-124 125-129 130-134 135-139 140-149 150-203 204-255 | Color 1 Color 1 + Color 2 Color 2 Color 2 + Color 3 Color 3 Color 3 + Color 4 Color 4 Color 4 + Color 5 Color 5 Color 5 + Color 6 Color 6 Color 6 + Color 7 Color 7 Color 7 + Color 8 Color 8 Color 8 + Color 9 Color 9 Color 9 + Color 10 Color 10 Color 10 + Color 11 Color 11 Color 11 + Color 12 Color 12 Color 12 + Color 13 Color 13 Color 13 + Color 14 Color 14 Color 14 + Color 1 Forward flow (from fast to slow) Reverse flow (from slow to fast) |
| 2 | Strobe | 0 - 3 4 -103 104-107 108-207 209-212 213-251 252-255 | closed light Pulse strobe (from slow to fast) open light Effect strobe (slow to fast) open light Random strobe (slow to fast) open light |
| 3 | Dimming | 0-255 | From dark to light |
| 4 | pattern plate | 0 - 5 6 - 11 12 - 17 18 - 23 24 - 29 30 - 35 36 - 41 42 - 47 48 - 53 54 - 59 60 - 65 66 - 71 72 - 77 78 - 83 84 - 89 90-134 135-139 140-185 186-190 191-195 196-200 201-205 206-210 211-215 216-220 221-225 226-230 231-235 236-240 241-245 246-250 251-255 | Solid Figure 1 Solid Figure 2 Solid Figure 3 Solid Figure 4 Solid Figure 5 Solid Figure 6 Solid Figure 7 Solid Figure 8 Solid Figure 9 Solid Figure 10 Solid Figure 11 Fixed Figure 12 Fixed Figure 13 Fixed Figure 14 Fixed Figure 15 Forward flow (from fast to slow) Solid Figure 1 Reverse flow (from slow to fast) Figure 2 Jitter (from slow to fast) Figure 3 Jitter (from slow to fast) Figure 4 Jitter (from slow to fast) Figure 5 Jitter (from slow to fast) Figure 6 Jitter (from slow to fast) Figure 7 Jitter (from slow to fast) Figure 8 Jitter (from slow to fast) Figure 9 Jitter (from slow to fast) Figure 10 Jitter (from slow to fast) Figure 11 Jitter (from slow to fast) Figure 12 Jitter (from slow to fast) Figure 13 Jitter (from slow to fast) Figure 14 Jitter (from slow to fast) Figure 15 Jitter (from slow to fast) |
| 5 | Prism 1 | 000-127 128-255 | Prism 1 pop-up Prism 1 cut-in |
| 6 | Prism 1 rotation | 000-064 065-127 128-191 192-255 | Index Forward running water (from slow to fast Reverse flow (from slow to fast) Positive and negative turn (from slow to fast) |
| 7 | Prism 2 | 000-127 128-255 | Prism 2 pop-up Prism 2 Cut |
| 8 | Prism 1 rotation | 000-064 065-127 128-191 192-255 | Index Forward running water (from slow to fast Reverse flow (from slow to fast) Positive and negative turn (from slow to fast) |
| 9 | Focus | 000-255 | Pattern clarity from far to near |
| 10 | X Axis | 000-255 | Horizontal 540 degree scanning |
| 11 | X Axis Trim | 000-255 | Horizontal 1.2 Trim |
| 12 | Y axis | 000-255 | Vertical 270 degree scan |
| 13 | Y-axis Trim | 000-255 | Vertical 1.2 Trim |
| 14 | XY speed | 000-255 | From fast to slow |
| 15 | atomization | 0-255 | 128-255 Atomizer Cut-in |
| 16 | Colorful | 0-255 | 0-127 colorful mirror pops up 128-255 Colorful Mirror Cut In |
| 17 | Lamp ignition and lamp extinguishing | 0-255 | 0-99 No effect 100-105 Turn off the bulb 106-199 No effect 200-205 light bulb 206-255 No effect |
| 18 | Reset | 0-255 | 0-249 No effect 250-255 Lamp Reset |

CH16 (Simple Channel Table)

| Channel | Channel Value | Function |
| --- | --- | --- |
| 1 | 0-255 | Color Wheel |
| 2 | 0-255 | Cut light/strobe |
| 3 | 0-255 | Dimming |
| 4 | 0-255 | pattern plate |
| 5 | 0-255 | Prism 1 |
| 6 | 0-255 | Prism 12 Rotation |
| 7 | 0-255 | Prism 2 |
| 8 | 0-255 | atomization |
| 9 | 0-255 | Focus |
| 10 | 0-255 | X |
| 11 | 0-255 | X Trim |
| 12 | 0-255 | Y |
| 13 | 0-255 | Y Trim |
| 14 | 0-255 | XY speed |
| 15 | 0-255 | Colorful |
| 17 | 0-255 | Bulb Control/Reset |

CH18 (Simple Channel Table)

| Channel | Channel Value | Function |
| --- | --- | --- |
| 1 | 0-255 | Color Wheel |
| 2 | 0-255 | Cut light/strobe |
| 3 | 0-255 | Dimming |
| 4 | 0-255 | pattern plate |
| 5 | 0-255 | Prism 1 |
| 6 | 0-255 | Prism 1 Rotation |
| 7 | 0-255 | Prism 2 |
| 8 | 0-255 | Prism 2 rotation |
| 9 | 0-255 | Focus |
| 10 | 0-255 | X |
| 11 | 0-255 | X Trim |
| 12 | 0-255 | Y |
| 13 | 0-255 | Y Trim |
| 14 | 0-255 | XY speed |
| 15 | 0-255 | atomization |
| 16 | 0-255 | Colorful |
| 17 | 0-255 | Bulb Control |
| 18 | 0-255 | Reset |

Product Warranty Card

Product Name ----------------------- Product Model----------------------

Date of Manufacture---------------------

| End User: |  |
| --- | --- |
| Unit Name: | Contact: |
| Address: | Telephone: |
| Sales shop |  |
| Contact: | Telephone: |

Packing size: 58cm * 43cm * 41cm
Gross weight: 15.6KG