RH-H960Strobe lamp manual

As the products are constantly updated, the contents of this manual may be slightly different from the actual products. Please understand!

Product catalog

1、 Display panel key function

2、 Menu function description

3、 Channel description

4、 Use of common functions

5、 Master slave settings

1： Display panel operation

Menu up and down confirmation

Basic usage:

Menu key: select various functions

Up: Select function parameters incrementally

Down: function parameter decreasing selection

Confirm (): confirm the current menuENTER

2、 Menu function

After power on, press the menu key (menu key) to successively display the following functions, and then press the OK key to confirm.(this sequence is a new machine sequence. If the lamp has used other modes and saved this mode before power off, the mode before last power off will be displayed after power on again). After setting other functions, you also need to press the save key to save, otherwise the effect of setting before power off will not appear after power on next time

| D001 |  | 001-512 | Address code |
| --- | --- | --- | --- |
| 11CH | Channel mode |  | Channel selection: 05ch, 11ch, 336ch, 347ch |
| -512 | Operation mode |  | DMX512, voice control, self-propelled |
| C000 | Color selection | C000-C007 | Color selection function |
| E000 | Effect mode | E000-E033 | Effect selection (312) |
| P000 | Effect speed | P000-P255 | Press up / down to select the running speed (p000-p255), and press enter to save the current running speed. |
| R255 | Manual red | R000-R255 | Manual red adjustment. When there is no console, use the up / down key to adjust the brightness between r000-r255, |
| G255 | Manual green | G000-G255 | Manual green adjustment. When there is no console, use the up / down key to adjust the brightness between g000-g255 |
| B255 | Manual blue | B000-B255 | Manual blue adjustment. When there is no console, use the up / down key to adjust the brightness between b000-b255 |
| F000 | Manual stroboscopic | F000-F255 | Use the up or down key to select the strobe speed |
| T000 | Temperature display | T000-T105 | Display temperature |
| After setting, press the OK key to save, and the display will flash once when saving, |  |  |  |

3、 Channel description

| 5 channels | Channel value | function |
| --- | --- | --- |
| 1 | 000-255 | Total dimming |
| 2 | 000-255 | 0-9Stroboscopic no effect 10-255Stroboscopic from slow to fast |
| 3 | 000-255 | The red brightness is adjusted from small to large |
| 4 | 000-255 | Green brightness is adjusted from small to large |
| 5 | 000-255 | Blue brightness is adjusted from small to large |

| 11 channels | Channel value | function |
| --- | --- | --- |
| 1 | 000-255 | Total dimming |
| 2 | 000-255 | 0-9Stroboscopic no effect 10-255Stroboscopic from slow to fast |
| 3 | 000-255 | Red brightness |
| 4 | 000-255 | Green brightness |
| 5 | 000-255 | Blue brightness |
| 6 | 000-255 | Macro function |
| 7 | 000-255 | Macro function speed |
| 8 | 000-255 | Effect color selection |
| 9 | 000-255 | Static effect selection |
| 10 | 000-255 | Dynamic effect selection |
| 11 | 000-255 | Effect speed |

| 336 channels | Channel value | function |
| --- | --- | --- |
| 1 | 0-255 | Red (group 1 light beads) |
| 2 | 0-255 | Green (group 1 light beads) |
| 3 | 0-255 | Blue (group 1 light beads) |
| 4 | 0-255 | Red (group 2 light beads) |
| 5 | 0-255 | Green (group 2 light beads) |
| 6 | 0-255 | Blue (group 2 light beads) |
| 7 | 0-255 | Red (Group 3 light beads) |
| 8 | 0-255 | Green (Group 3 light beads) |
| 9 | 0-255 | Blue (Group 3 light beads) |
| 10 | 0-255 | Red (Group 4 light beads) |
| 11 | 0-255 | Green (Group 4 light beads) |
| 12 | 0-255 | Blue (Group 4 light beads) |
| 13 | 0-255 | Red (Group 5 light beads) |
| 14 | 0-255 | Green (Group 5 light beads) |
| 15 | 0-255 | Blue (Group 5 light beads) |
| 16 | 0-255 | Red (Group 6 light beads) |
| 17 | 0-255 | Green (Group 6 light beads) |
| 18 | 0-255 | Blue (Group 6 light beads) |
| 19 | 0-255 | Red (Group 7 light beads) |
| 20 | 0-255 | Green (Group 7 light beads) |
| 21 | 0-255 | Blue (Group 7 light beads) |
| ....... |  |  |
| 334 | 0-255 | Red (Group 112 light beads) |
| 336 | 0-255 | Green (Group 112 light beads) |
| 336 | 0-255 | Blue (Group 112 light beads) |

| 347 channel | Channel value | function |
| --- | --- | --- |
| 1 | 000-255 | Total dimming |
| 2 | 000-255 | 0-9Stroboscopic no effect 10-255Stroboscopic from slow to fast |
| 3 | 000-255 | The red brightness is adjusted from small to large |
| 4 | 000-255 | Green brightness is adjusted from small to large |
| 5 | 000-255 | Blue brightness is adjusted from small to large |
| 6 | 000-255 | Macro function |
| 7 | 000-255 | Macro function speed |
| 8 | 000-255 | Effect color selection |
| 9 | 000-255 | Static effect |
| 10 | 000-255 | Dynamic effect |
| 11 | 000-255 | Effect speed |
| 12 | 0-255 | Red (group 1 light beads) |
| 13 | 0-255 | Green (group 1 light beads) |
| 14 | 0-255 | Blue (group 1 light beads) |
| 15 | 0-255 | Red (group 2 light beads) |
| 16 | 0-255 | Green (group 2 light beads) |
| 17 | 0-255 | Blue (group 2 light beads) |
| 18 | 0-255 | Red (Group 3 light beads) |
| 19 | 0-255 | Green (Group 3 light beads) |
| 20 | 0-255 | Blue (Group 3 light beads) |
| 21 | 0-255 | Red (Group 4 light beads) |
| 22 | 0-255 | Green (Group 4 light beads) |
| 23 | 0-255 | Blue (Group 4 light beads) |
| 24 | 0-255 | Red (Group 5 light beads) |
| 25 | 0-255 | Green (Group 5 light beads) |
| 26 | 0-255 | Blue (Group 5 light beads) |
| 27 | 0-255 | Red (Group 6 light beads) |
| 28 | 0-255 | Green (Group 6 light beads) |
| 29 | 0-255 | Blue (Group 6 light beads) |
| 30 | 0-255 | Red (Group 7 light beads) |
| 31 | 0-255 | Green (Group 7 light beads) |
| 32 | 0-255 | Blue (Group 7 light beads) |
| ..... |  |  |
| 345 | 0-255 | Red (Group 112 light beads) |
| 346 | 0-255 | Green (Group 112 light beads) |
| 347 | 0-255 | Blue (Group 112 light beads) |

4、 Use of common functions

1. DMX512 control

After power on, set the address code. If it is the first set, set the address code to 001. Connect the DMX512 console with a three core signal line, and the digital display tube will stop flashing, indicating that the 512 signal has entered the lamp, and the lamp can be controlled.At this time, use the console to control relevant functions according to the instructions of the channel table.

When there are multiple lamps, as long as the rear lamps are connected with the front lamps with signal wires and the address code is set, at this time, the nixie tube display board will stop flashing because the rear lamps are connected with 512 signal, and then it can be controlled like the front lamps.

2，Other functions can be used as long as the relevant functions are pressed through the menu key (select the corresponding speed with the up key or dowm key)

5、 Master slave setting

2Two or more identical lamps are connected by 3-core signal wires. If any host is set as voice control or gradual change, pulse change and jump change, and other lamps are set in 001 mode, it will enter the master-slave mode and the slave display screen will not flash (flashing indicates that the machine has not received a signal, and the connecting wire must be checked)

Special note: only one host can be set in a group of lamps. If there are multiple hosts, all lamps will flash disorderly and out of synchronization.