Rotating retro lamp

1. working voltage: AC220V/50Hz.

2, product power: 250W three 60W golden light

36 3535 three-in-one lamp beads (3W one) and one 40W four-in- one lamp bead

- Control mode: DMX512, 15 channels, 20 channels and 26 channels.

- Working mode: DMX512, master/slave, self-propelled, voice contr

- Dimming mode: 0~100% ultra-smooth dimming.

- Strobe: 25 times/second

| channe | function | DMX value | describe |
| --- | --- | --- | --- |
| 1 | X axis | 0-255 | 0~360 degrees |
| 2 | X Wuji | 0-49 | 0-360 degree rotation (fast to slow) X axis speed e |
|  |  | 50-152 | Clockwise stepless rotation (from fast to slow) |
|  |  | 153-255 | Counter-clock stepless rotation (from slow to fast) |
| 3 | otal dimming of golden ligh | 0-255 | Linear dimming from dark to light |
| 4 | stroboscopic | 0 | No function |
|  |  | 1-255 | Synchronous strobe speed from slow to fast (1HZ~25H |
| 5 | Golden light | 0-4 | Extinguish light |
|  |  | 5-41 | Golden light 1 |
|  |  | 42-76 | Golden light 2 |
|  |  | 77-112 | Golden light 3 |
|  |  | 113-148 | Golden light 1+ golden light 2 |
|  |  | 149-184 | Golden light 2+ golden light 3 |
|  |  | 185-220 | Golden light 2+ golden light 3 |
|  |  | 221-255 | Golden light 1+ golden light 2+ golden light 3 |
| 6 | Dyeing total dimming |  | Linear dimming from dark to light |
| 7 | stroboscopic | 0 |  |
|  |  | 1-255 | Synchronous strobe speed from slow to fast (1HZ~25H |
| 8 | 1 (dyeing effect running co | 0-4 | Extinguish light |
|  |  | 5-41 | red |
|  |  | 42-76 | green |
|  |  | 77-112 | blue |
|  |  | 113-148 | Red+green |
|  |  | 149-184 | Red+blue |
|  |  | 185-220 | Green+blue |

|  |  | 221-255 | Red+Green+Blue |
| --- | --- | --- | --- |
| 9 | g 2 (base color of dyeing ef | 0-4 | Extinguish light |
|  |  | 5-41 | red |
|  |  | 42-76 | green |
|  |  | 77-112 | blue |
|  |  | 113-148 | Red+green |
|  |  | 149-184 | Red+blue |
|  |  | 185-220 | Green+blue |
|  |  | 221-255 | Red+Green+Blue |
| 10 | Dyeing 3 | 0-4 | Extinguish light |
|  |  | 5-41 | red |
|  |  | 42-76 | green |
|  |  | 77-112 | blue |
|  |  | 113-148 | Red+green |
|  |  | 149-184 | Red+blue |
|  |  | 185-220 | Green+blue |
|  |  | 221-255 | Red+Green+Blue |
| 11 | beam | 0-4 | Extinguish light |
|  |  | 5-36 | red |
|  |  | 37-48 | green |
|  |  | 48-79 | blue |
|  |  | 80-111 | white |
|  |  | 112-144 | Red+green |
|  |  | 145-176 | Red+blue |
|  |  | 177-208 | Green+blue |
|  |  | 209-240 | Red++Green+Blue |
|  |  | 241-255 | Red++Green+Blue+White |
| 12 | golden light) macro functio | 0-4 | neutrality |
|  |  | 5-85 86-170 171-255 | Built-in effect 1 (from slow to fast) Built-in effect 2 (from slow to fast) Built-in effect 3 (from slow to fast) |
| 13 | dyeing light) macro functio | 0-4 | neutrality |
|  |  | 4-45 | Built-in effect 1 (from slow to fast) |
|  |  | 46-87 | Built-in effect 2 (from slow to fast) |
|  |  | 88-129 | Built-in effect 3 (from slow to fast) |
|  |  | 130-171 | Built-in effect 4 (from slow to fast) |
|  |  | 172-213 | Built-in effect 5 (from slow to fast) |
|  |  | 213-255 | Built-in effect 6 (from slow to fast) |
| 14 | (beam) macro function | 0-4 | neutrality |
|  |  | 5-85 86-170 171-255 | Built-in effect 1 (from slow to fast) Built-in effect 2 (from slow to fast) Built-in effect 3 (from slow to fast) |
| 15 reset |  | 0-199 200-255 | neutrality reset |

| channe | function | DMX value | describe |
| --- | --- | --- | --- |
| 1 | X axis | 0-255 | 0~360 degrees |
| 2 | X Wuji | 0-49 | 0-360 degree rotation (fast to slow) X axis speed e |
|  |  | 50-152 | Clockwise stepless rotation (from fast to slow) |
|  |  | 153-255 | Counter-clock stepless rotation (from slow to fast) |
| 3 | otal dimming of golden ligh | 0-255 | Linear dimming from dark to light |
| 4 | stroboscopic | 0 | No function |
|  |  | 1-255 | Synchronous strobe speed from slow to fast (1HZ~25H |
| 5 | Golden light 1 Golden light 2 Golden light 3 Dyeing total dimming Dyeing stroboscopic | 0-255 | Linear dimming from dark to light |
| 6 |  | 0-255 | Linear dimming from dark to light |
| 7 |  | 0-255 | Linear dimming from dark to light |
| 8 |  | 0-255 | Linear dimming from dark to light |
| 9 |  | 0 | No function |
|  |  | 1-255 | Synchronous strobe speed from slow to fast (1HZ~25H |
| 10 11 12 13 14 15 16 | light (dyeing effect runnin ight (background color of dy Dyed blue light Beam red light Beam green light Blue light beam Beam white light | 0-255 | Linear dimming from dark to light |
|  |  | 0-255 | Linear dimming from dark to light |
|  |  | 0-255 | Linear dimming from dark to light |
|  |  | 0-255 | Linear dimming from dark to light |
|  |  | 0-255 | Linear dimming from dark to light |
|  |  | 0-255 | Linear dimming from dark to light |
|  |  | 0-255 | Linear dimming from dark to light |
| 17 | golden light) macro functio | 0-4 | neutrality |
|  |  | 5-85 86-170 171-255 | Built-in effect 1 (from slow to fast) Built-in effect 2 (from slow to fast) Built-in effect 3 (from slow to fast) |
| 18 | dyeing light) macro functio | 0-4 | neutrality |
|  |  | 4-45 | Built-in effect 1 (from slow to fast) |
|  |  | 46-87 | Built-in effect 2 (from slow to fast) |
|  |  | 88-129 | Built-in effect 3 (from slow to fast) |
|  |  | 130-171 | Built-in effect 4 (from slow to fast) |
|  |  | 172-213 | Built-in effect 5 (from slow to fast) |
|  |  | 213-255 | Built-in effect 6 (from slow to fast) |
| 19 | (beam) macro function | 0-4 | neutrality |
|  |  | 5-85 86-170 171-255 | Built-in effect 1 (from slow to fast) Built-in effect 2 (from slow to fast) Built-in effect 3 (from slow to fast) |
| 20 reset |  | 0-199 200-255 | neutrality reset |

| 6 channe | function | DMX value | describe |
| --- | --- | --- | --- |
| 1 | X axis | 0-255 | 0~360 degrees |
| 2 | X Wuji | 0-49 | 0-360 degree rotation (fast to slow) X axis speed e |
|  |  | 50-152 | Clockwise stepless rotation (from fast to slow) |
|  |  | 153-255 | Counter-clock stepless rotation (from slow to fast) |
| 3 | otal dimming of golden ligh | 0-255 | Linear dimming from dark to light |
| 4 | stroboscopic | 0 | No function |
|  |  | 1-255 | Synchronous strobe speed from slow to fast (1HZ~25H |
| 5 | Golden light 1 Golden light 2 Golden light 3 Dyeing total dimming Dyeing stroboscopic | 0-255 | Linear dimming from dark to light |
| 6 |  | 0-255 | Linear dimming from dark to light |
| 7 |  | 0-255 | Linear dimming from dark to light |
| 8 |  | 0-255 | Linear dimming from dark to light |
| 9 |  | 0 | No function |
|  |  | 1-255 | Synchronous strobe speed from slow to fast (1HZ~25H |
| 10 11 12 13 14 15 16 17 18 19 20 21 22 | ight 1 (dyeing effect runni ight 1 (dyeing effect backg Dyed blue light 1 Dyed red light 2 Dyed green light 2 Dyed blue light 2 Dyed red light 3 Dyed green light 3 Dyed blue light 3 Beam red light Beam green light Blue light beam Beam white light | 0-255 | Linear dimming from dark to light |
|  |  | 0-255 | Linear dimming from dark to light |
|  |  | 0-255 | Linear dimming from dark to light |
|  |  | 0-255 | Linear dimming from dark to light |
|  |  | 0-255 | Linear dimming from dark to light |
|  |  | 0-255 | Linear dimming from dark to light |
|  |  | 0-255 | Linear dimming from dark to light |
|  |  | 0-255 | Linear dimming from dark to light |
|  |  | 0-255 | Linear dimming from dark to light |
|  |  | 0-255 | Linear dimming from dark to light |
|  |  | 0-255 | Linear dimming from dark to light |
|  |  | 0-255 | Linear dimming from dark to light |
|  |  | 0-255 | Linear dimming from dark to light |
| 23 | golden light) macro functio | 0-4 | neutrality |
|  |  | 5-85 86-170 171-255 | Built-in effect 1 (from slow to fast) Built-in effect 2 (from slow to fast) Built-in effect 3 (from slow to fast) |
| 24 | dyeing light) macro functio | 0-4 | neutrality |
|  |  | 4-45 | Built-in effect 1 (from slow to fast) |
|  |  | 46-87 | Built-in effect 2 (from slow to fast) |
|  |  | 88-129 | Built-in effect 3 (from slow to fast) |
|  |  | 130-171 | Built-in effect 4 (from slow to fast) |
|  |  | 172-213 | Built-in effect 5 (from slow to fast) |
|  |  | 213-255 | Built-in effect 6 (from slow to fast) |
| 25 | (beam) macro function | 0-4 | neutrality |
|  |  | 5-85 86-170 171-255 | Built-in effect 1 (from slow to fast) Built-in effect 2 (from slow to fast) Built-in effect 3 (from slow to fast) |
| 26 reset |  | 0-199 200-255 | neutrality reset |

| serial number | English display | explain |
| --- | --- | --- |
| 1 | ADDR | A001---A512 address code |
| 2 | MANU | Operation mode selection |
| 3 | RUN | control) |
| 4 | CHND | 15CH, 20CH and 26CH channel selection |
| 5 | SOUD | ON, OFF sound control switch |
| 6 | LED | ON, OFF background light switch |
| 7 | DISP | YES, NO reverse display |
| 8 | PSET | YES, NO motor x reverse |
| 9 | TILT | YES, NO motor y reverse |
| 10 | VER | V101 software version number |
| 11 | FAET | YES, NO restore factory settings. |