1. Display the panel and the key definition

Menu confirms up and down

Order key: selection function

Up key:, the parameter add

Down key:, parameter parameter

Confirmation key: determine and save

2. Menu function

After power on, press the menu button to display the menu menu; modify the function parameters up or down, and save the current function and parameters (with power loss memory after saving).

Menu menu:

| A001 |  | A512 | Modify the address code (A001~A 512), confirm the key save, default A001. |
| --- | --- | --- | --- |
| CH 04 |  | CH152 | Switch up or down ch04, ch16, and, ch168, six channels, confirmation key save, default CH 15. |
| P000 |  | P255 | stroboflash |
| M009 |  | M063 | Pattern channel RGB mode |
| S000 |  | S255 | Speed channel |
| n000 |  | N255 | Mode channel W mode |
| S000 |  | S255 | Speed channel |
| Q000 |  | Q063 | Pattern channel RGBW mode |
| S000 |  | S255 | Speed channel |
| R255 |  | R000 | Modify the red bead brightness (R000~ R255) and save the key, default R255. |
| G255 |  | G000 | Modify the green bead brightness (G000~ G255), confirm the key save, default G255. |
| B255 |  | B000 | Modify the blue bead brightness up or down (B000~ B255), save the key, default B255. |
| W255 |  | W000 | Change the white bead brightness (W000~ W255), confirm the key save, default W255. |
| T000 |  |  | Display temperature, if T045 means the current lamp temperature is 45℃; 10K thermistor is not installed, display T000. |

3. Master-slave machine control

Two or more identical lamps are connected with DMX three-core signal line, the lamps are set to A001~A512 any address code, set as the host, other lamps are slave, all the slave display is not flashing; when the host gradient, pulse change, jump and self-walk effect.

Special attention: 1, a group of lamps can only be set with one host, if there are more than one host, all the lamps will flash and not synchronized.

2. All lamps must be used when the DMX512 console is closed.

4. Factory setting

At any address code of A001~A512, press the menu key for 3 seconds to enter the factory setting. The factory setting is mainly based on the functions of the output power of the lamps, the fan setting mode, setting the temperature protection point and the sending parameters. In the factory setting mode, press the menu button for 3 seconds to exit.

Factory Settings Table:

| R 255 |  | R032 | Up or down modify red bead current (R032-R 255), confirm key save, default R245. |
| --- | --- | --- | --- |
| G 255 |  | G032 | Up or down modify green bead current (R032-R 255), confirm key save, default R245. |
| B 255 |  | B032 | Change the blue bead current up or down (R032-R 255), confirm key save, default R245. |
| W255 |  | W 032 | Up or down modify white bead current (R032-R 255), confirm key save, default R245. |
| FAN0 |  | FAN1 | F Fan setting: FAN 0 lamp bead is bright to start the fan, FAN 1 reaches the set temperature protection point to start the fan, confirm the key to save. |
| T040 |  | T 070 | Set the temperature protection point, modify the parameters up or down (40℃ ~70℃), press the confirmation key to save, default T065. |
| S end |  | S end | Send the factory setting parameters up or down to all other lamps connected in parallel with three-core signal lines; confirm the sending parameters to press the menu button for 3 seconds, and press the confirmation button to cancel the sending. |

5. DMX512 Console

After power-on, all the lamps' address codes are set, and all the lamps are connected to the DMX512 console in parallel with three-core signal lines. The address code will stop flashing, indicating that the DMX512 console signal has been sent to the lamps, and control relevant functions with the DMX512 console according to the instructions of each channel.

CH4 Channel description:

| channel | The channel value | basic function |
| --- | --- | --- |
| 1 | 000-255 | R |
| 2 | 000-255 | G |
| 3 | 000-255 | B |
| 4 | 000-255 | W |

CH 15 channel description: Default

| channel | The channel value | basic function |
| --- | --- | --- |
| 1 | 000-255 | aiming |
| 2 | 000-255 | R |
| 3 | 000-255 | G |
| 4 | 000-255 | B |
| 5 | 000-255 | W |
| 6 | 000-255 | pigment |
| 7 | 000-255 | RGB pattern |
| 8 | 000-255 | RGB velocity |
| 9 | 000-255 | W pattern |
| 10 | 000-255 | W velocity |
| 11 | 000-255 | RGBW stroboflash |
| 12 | 000-255 | RGBW pattern |
| 13 | 000-255 | RGBW velocity |
| 14 | 000-255 | background colour |
| 15 | 000-255 | Background color light |

CH152 Channel Description:

| channel | The channel value | basic function |
| --- | --- | --- |
| 1 | 000-255 | Paragraph 01 red lamp bead linear dimming. |
| 2 | 000-255 | Paragraph 01 green lamp bead linear dimming. |
| 3 | 000-255 | Paragraph 01 with linear dimming in blue lamp beads. |
| 5 | 000-255 | Paragraph 02 red lamp bead linear dimming. |
| 6 | 000-255 | Paragraph 02: linear dimming of green lamp beads. |
| 7 | 000-255 | Paragraph 02 Blue lamp bead linear dimming. |
| … | …… | …… |
| 142 | 000-255 | Paragraph 48 red lamp bead linear dimming. |
| 143 | 000-255 | Paragraph 48 Green lamp bead linear dimming. |
| 144 | 000-255 | Paragraph 48: linear dimming in blue lamp beads. |
| … | …… | …… |
| 145 | 000-255 | Paragraph 1 white lamp bead linear dimming. |
| 146 | 000-255 | Paragraph 2 of the white lamp bead with linear dimming. |
| 147 | 000-255 | Paragraph 3 of the white lamp bead with linear dimming. |
| 148 | 000-255 | Paragraph 4 white lamp bead linear dimming. |
| 149 | 000-255 | Paragraph 5 white lamp bead linear dimming. |
| 150 | 000-255 | Paragraph 6 of the white lamp bead with linear dimming. |
| 151 | 000-255 | Paragraph 7 white lamp bead linear dimming. |
| 152 | 000-255 | Paragraph 8 of the white lamp bead linear dimming. |

Vi. Technical parameters:

Voltage: AC100~240V 50 / 60 HZ

Power: 120W

Pearl: 768 RGB beads, 96 W beads

Control mode: DMX512, self-go, master-slave, with RDM function

Channel: CH4, CH15, and CH 152

Dimming: 16bit 0~100% Linear dimming

Features: horse racing + explosive flash

Operating temperature: -30 degrees ~50 degrees

Floribe frequency: 1-30 HZ

Appearance: metal, black

Connection mode: DMX512 input and output

/ Power supply input / output.

IP grade: IP20

Size: Gross weight: