RH-B36S

36  LEDS WASH MOVING HEAD

Please read the instructions carefully before use.

Product technical parameters:

Voltage: AC90-240v

Work rate: 400w

Frequency rate: 50-60Hz

Lamp beads: single 4 and 1 lamp beads

Access: 24CH / 16CH

Control mode: DMX/ master-slave / voice control

Horizontal scan: 540 °(16-bit accurate scan)

Vertical scan: 270 (16-bit accurate scan)

Stroboscopic: 1-25 per second or random

Display screen: LCD / English

Net weight: 7 kg.

Ruler: 38*31*46cm (carton 1in1)

1 address Set (DMX address setting)

This option is used to set the DMX address, press the "Enter" key to enter the setting, press the "UP" key to increase the address number, "DOWN" key can be the address number decline, select the address and press "Enter" to save the exit.

2U run Mode (Lamp working Mode setting)

This option is used to set the working mode of the lamp body, press the "Enter" key to enter the setting, or press the "UP" or "DOWN" key to select the working mode, which is followed by "DMX512", "Slow", "Fast", "Sound", and "Slave". Select work mode and press "Enter" to save exit.

DMX512: control station operation mode.

Slow: slow self-walking mode

Fast: fast self-walking mode

Sound sound control self-walking mode

Slave: slave mode

3 convert PAN (X axis reverse setting)

This option is to reverse the direction of the X axis, press the "Enter" key to enter the setting, press the "UP" key or "DOWN" key to select the X axis in reverse, select "Yes" to indicate the direction of the X axis, and select "NO" to indicate that the X-axis is not oriented. Select the direction and press "Enter" to save the exit.

4v convert TILT (Y axis reverse setting)

This option is to set the Y axis in reverse, press the "Enter" key to enter the setting, press the "UP" key or "DOWN" key to select the Y axis in reverse, select "Yes" to indicate the Y-axis direction, and select "NO" to indicate that the Y-axis is not oriented. Select the direction and press "Enter" to save the exit.

5 Display Set (LCD backlight setting)

This option is to display panel LCD backlight switch settings, press "Enter" key to enter settings, press "UP" or "DOWN" key to select backlight switch, select "Yes" to indicate that the LCD backlight has been long, not turned off, Select "NO" to indicate that no key operation is turned off for 20 seconds until it reopens. Select the mode and press "Enter" to save the exit.

6, Sound Sence (sound control sensitivity adjustment)

This option is to switch the lamp body voice sensitivity, press the "Enter" key to enter the setting, press the "UP" key or "DOWN" key to select the sensitivity of 0 ~ 100 degrees, select the sensitivity, then press "Enter" to save.

7, Factory Default (reverts to factory setting)

Users choose to restore factory settings, so the parameters will be set according to the product before the factory.

7t reset (lamp reset)

After the user chooses to reset, wait 3 seconds, the lamp body will be reset.

8 XY-Init Adjust (XY axis initial position fine-tuning setting)

After the user presses the "MENU" key for 5 seconds, the user will enter the XY axis initial position fine adjustment setting, press the "UP" key or "DOWN" key to adjust the initial position, determine the initial position and press "Enter" to save the exit.

9 White Balance (LED white balance setting)

After the user presses the “MENU” button for 5 seconds, it will enter the LED white balance setting. Press the “UP” button or the “DOWN” button to adjust the brightness of the red, blue, white and white colors. The options are “SET_R” and “SET_G” respectively. ""SET_B""SET_W", after confirming the color brightness, press "Enter" to save and exit.

15 channel mode:

Aisle

1  X axis (horizontal) 00255

2  Y axis (vertical) 000255

3  XY speed 000-255

4  total dimming  000-255

5  red (R first group  000255

6  Green G Group One| 000255

7  blue (B) the first group  00255

8  white (W first group  000255

9  stroboflash  000-255

10  focusing  000-255

11  function mode  000255

12  function mode speed|  000255

13  x axis fine-tuning  000255

14  Y-axis fine-tuning  000-255

15  Reset  1000249  invalid  250-255 reset

Thank you for using the products produced by our company for your safety

Fully effective use of this product

Before, please read this manual carefully and completely

Book. This manual contains: the performance of the product is very true

Minghe how to install it safely, and how important it is to use

For information, please keep it in a safe place for reference when using

Please strictly follow the relevant instructions when installing and using